# IoT Milk Preservation System Documentation

# Wiring Diagram (Textual Description)

ESP32 milk preservation — wiring diagram

DHT22 sensor — use a 10kΩ resistor between DATA and VCC (pull-up):

- Pin 1 (VCC) → ESP32 3V3

- Pin 2 (DATA) → GPIO4 (with 10kΩ pull-up to 3V3)

- Pin 4 (GND) → GND

LCD 16×2 with I²C backpack — only 4 wires needed:

- VCC → ESP32 VIN (5V)

- GND → GND

- SDA → GPIO21

- SCL → GPIO22

Relay module:

- VCC → VIN (5V)

- GND → GND

- IN → GPIO26

- COM/NO → your cooling unit (fridge/Peltier)

Buzzer & LED — both share GPIO27:

- Buzzer (+) → GPIO27, Buzzer (–) → GND

- LED (+) → GPIO27 via a 220Ω resistor, LED (–) → GND

Check summary if correctly connected

ESP32 → DHT22 Sensor: Pin 4 → Data, 3.3V → VCC, GND → GND

ESP32 → LCD (I²C): SDA (GPIO 21) → SDA, SCL (GPIO 22) → SCL, 5V → VCC, GND → GND

ESP32 → RGB LED: GPIO 26 → Red, GPIO 27 → Green, GPIO 14 → Blue, common cathode → GND

ESP32 → Relay: GPIO 33 → IN, 5V → VCC, GND → GND

ESP32 → Buzzer: GPIO 25 → +, GND → -

# 2. Flow Chart (Logic Overview)

Start / Setup

│

┌───────▼────────┐

│ Read Temp & Hum│

└───────┬────────┘

│

┌─────────▼─────────┐

│ Sensor Error?     │

└───────┬───────────┘

│Yes

▼

Display "Sensor Error"

Turn OFF all outputs

│

└───────────────► Loop

│No

▼

┌───────────────────────────────┐

│ Check Temperature Thresholds  │

└───────────────────────────────┘

Blue ≤2°C → Too Cold

Red 5–8°C → Too Hot

Flash Red >8°C → Critical Temp

│

▼

┌───────────────────────────────┐

│ Check Humidity Thresholds     │

└───────────────────────────────┘

Yellow <70% → Low Humidity

Green 70–79% → Moderate

Cyan 80–90% → Optimal

Purple 91–95% → High Humidity

Flash Purple/Red >95% → Critical Humidity

│

▼

┌─────────────-──┐

│ Update LCD     │

│ (Alternate T/H)│

└───────┬────-───┘

│

▼

Loop

# 3. Milk Test Criteria Documentation

| Test ID | Temp Range (°C) | Humidity Range (%) | System Status | LED Color | Relay | Buzzer | Meaning for Milk Preservation / Risks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TEST 1 | ≤ 2 | 80–90 | Too Cold | Blue | OFF | ON | Milk may freeze; ice crystals disrupt proteins. Overcooling risks poor texture. In cold‑room collection centers, improper chilling can mask adulteration or contamination. |
| TEST 2 | 3–5 | 80–90 | Safe Range | Cyan | OFF | OFF | Ideal preservation. Milk remains stable for pooling/processing. Automated monitoring ensures adulterated or contaminated milk is detected before mixing. |
| TEST 3 | > 5 | 80–90 | Too Hot | Red | ON | ON | Spoilage risk begins. Warm conditions accelerate bacterial growth. Unscrupulous farmers may exploit warm storage to dilute milk with water, compromising quality. |
| TEST 4 | > 8 | 80–90 | Critical Heat | Red Flash | ON | ON | High contamination danger. Milk should be isolated before mixing. At this stage, bacterial load is unsafe for infants and poses serious health risks. |
| TEST 5 | Any | < 70 | Low Humidity Warning | Yellow | OFF | OFF | Environment too dry; excessive ventilation reduces cooling efficiency. Poor storage conditions increase risk of adulteration detection failures. |
| TEST 6 | Any | 70–79 | Moderate Humidity | Green | OFF | OFF | Acceptable but below ideal. Continuous monitoring needed to prevent unnoticed contamination from sick cows or poor hygiene. |
| TEST 7 | Any | 80–90 | Optimal Humidity | Cyan | OFF | OFF | Best humidity for dairy preservation. Stable conditions support rapid, automated quality testing and accountability in supply chains. |
| TEST 8 | Any | > 90 | High Humidity Warning | Purple | OFF | ON | Excess moisture causes condensation, mold, and bacterial contamination. Mixing contaminated milk with good milk compromises entire batches, highlighting the need for automated rejection systems. |